BEAU-ROC MODEL DCS - DUMP BODY SPECIFICATIONS

DUMP BODY SPECIFICATIONS:

GENERAL:

Each bidder must indicate on the following items whether their bid is in compliance or not with the stated specifications.  Exceptions to the bid must be detailed on a separate written sheet using company letterhead.  Bidder must supply literature of the model being bid.  Successful bidder must consult with the supervisor on specifications before fabricating the body(ies).

MAKE & MODEL:

YES____  NO____

BEAU-ROC DCS (Diamond Side Medium Duty, no posts)

COMPLETION:

YES____  NO____

The unit(s) must be completed and delivered within ??? weeks after receipt of chassis.

DUMP BODY DIMENSIONS:

YES____  NO____

Length: ???'  (Options include 9' to 18' long, usually in 6" increments but 1" increments are acceptable as well)

YES____  NO____

Floor Width: 86" at the front of the body & 88" at the rear of the body

The body must be tapered to assist in load breakaway (no exceptions).

YES____  NO____

Side Height: ???" (Options include 24", 30", 36", 42”, 48”)

YES____  NO____

Tailgate Height: ???" (Options include 30”, 36", 42", 48", 54”, 60”)

YES____  NO____

Cabshield: ???" (Options include 12" to 48" in 6" increments)

DUMP BODY MATERIALS:

YES____  NO____

Floor:  ???

(Please choose one of the following:

7 gauge Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength)

1/4" Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength))

YES____  NO____

Front: 	1/8” Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength)

YES____  NO____

Sides: 	1/8” Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength)

YES____  NO____

Tailgate:  7 gauge Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength))

YES____  NO____

Longitudinals: ???

(Please choose one of the following:

For Bodies with a length of less than 13'-6" and less

Longitudinals shall be formed with 3/16” 44W (44,000 psi yield strength & 80,000 psi tensile strength)

For Bodies with a length of more than 13’-6”

Longitudinals shall be formed with 4 gauge 44W (44,000 psi yield strength & 80,000 psi tensile strength)

BODY UNDERSTRUCTURE DESIGN

YES____  NO____

Body shall be cross-memberless.  Cross-member understructure not acceptable, no exceptions.

YES____  NO____

YES____  NO____

Longitudinals shall be enclosed formed trapezoidal shaped with a minimum 1 3/4" return flanges on either side to support the floor with as wide a span as possible. Longitudinal shall have a bottom surface span of 3" wide.  Structural steel longitudinals not acceptable.

YES____  NO____

All seams shall be 100% fully welded.

YES____  NO____

Longitudinal Height: ???" (Options include 6", 8", 10", 12")

Body/Safety Prop:

(Please choose one of the following:

YES____  NO____

(For Bodies with an air tailgate)

Integrated Body/Safety Prop (No Exceptions) - Body Prop must be integral to the body understructure.  The body prop shall rotate down from the body and attach to the cradle when in use. Body Prop must meet the OSHA minimum requirement.

YES____  NO____

(For Bodies with a manual tailgate)

A single body prop that meets the OSHA requirement shall be provided with the body.

FLOOR DESIGN:

YES____  NO____

Shall be one-piece (no exceptions - multi-piece floors not acceptable).  Floor sheet shall be formed such that the floor bends up at 57 degrees to join the side sheet forming a knee brace.  The floor shall have a minimum flat width of 75’’ at front of the body and 77’’ at rear of the body.  All seams shall be 100% fully welded.

FRONT DESIGN:

YES____  NO____

Front Bulkhead shall be constructed of 2 overlapping pieces.  It shall have a formed recess, 12 1/4" deep by 17 1/4" wide, to house the trunnion mount hoist.  The formed recess shall have chamfered 45deg corners on the two front edges and the two rear edges to reduce stress points (no exceptions).

YES____  NO____

Front Bulkhead shall be designed with 7.5"x7.5" 45 degree front corners for reduced wind resistance and to aid in avoiding exhaust interference issues (no exceptions).

YES____  NO____

Front sheet to floor transition shall be a welded piece of steel at approximately 45 degrees for easy cleanout.

YES____  NO____

The steel used for the transition shall be a minimum of 7 gauge Hardox 450 steel (180,000 psi yield strength and 215,000 psi tensile strength) (no exceptions).

CABSHIELD DESIGN:

YES____  NO____

Cabshield shall be constructed of a minimum of 10 gauge steel.  Cabshield must be 100% fully welded (stitch welding not acceptable).

SIDES DESIGN:

YES____  NO____

Sides design shall include a formed side sheet plus a separate structural steel top rail & a separate formed rub rail.

YES____  NO____

Side shall be formed with a single pressed-in horizontal brace to increase rigidity (no exceptions).

Notes:

- "Weld-on horizontal bracing is not acceptable due to inferior design, more weight, and decreased flexibility of the side"

- "Flat sided designs are not acceptable due to inferior rigidity and stability of the side plate"

YES____  NO____

Lower rub rail shall be a minimum of 10 gauge steel, formed and welded to the side sheet and the floor.  The rub rail shall dirt shedding.  It shall have an angle of 40 degrees and shall have a 90 degree end face of 2 5/8" high (rub rails that finish in a point or rounded edge are not acceptable).

YES____  NO____

4"x3" top rail is formed and integral to the side sheet.

YES____  NO____

Each side shall consist of one front post, minimum 10 gauge steel and one rear post of one-piece construction and fabricated out of a minimum 3/16" steel.

YES____  NO____

Board pockets shall be 2.75" wide & fabricated with 3/16" steel.

YES____  NO____

Steps shall be 9.25" long x 1.125" round and installed on the driver side near the front of the body.  Formed bars are not acceptable.

YES____  NO____

All body seams must be 100% fully welded.

TAILGATE DESIGN:

YES____  NO____

Tailgate wear surface shall be a single piece, fully formed, with one pressed-in horizontal brace to increase rigidity. Flat panel designs are unacceptable due to inferior rigidity and stability.

YES____  NO____

Tailgate outer frame shall consist of single, dirt shedding, formed upper and lower brace.  Outer side bracing shall be 3"x4"x3/16" structural tubing.

Note:

- Weld-on intermediate horizontal and/or vertical braces are not acceptable due to the decrease in tailgate plate's resistance to deformation.

YES____  NO____

Top hardware will be top mount style with a minimum 3/4" thick hinge plate and 1.25" diameter x 3 ¾” length, tempered, removable, cold rolled steel pin.  The hinge shall have a tempered steel bushing for contacting the pin.

YES____  NO____

Top mount hinge shall have a 9" offset.

YES____  NO____

Bottom tailgate pins shall be a minimum of 1.25" in diameter and a minimum of 3" long.

YES____  NO____

Tailgate shall be double-acting.

YES____  NO____

Sufficient length of 3/8" Grade 70 chains for the full function of the tailgate.

YES____  NO____

All tailgate hinge linkage must have grease zerks at all pivot points.

YES____  NO____

All body seams must be 100% fully welded.

TAILGATE MATERIAL CHUTE:

YES____  NO____

Tailgate must be constructed with ??? (Choose one: one, two or three) material chutes.

(For Tailgate heights of 42" or more)

YES____  NO____

Material chute shall have fully open minimum dimensions of 13" wide by 13.5" high (no exceptions).

YES____  NO____

Material chute design must allow for the dispensing of small amounts of material without opening the entire tailgate.

YES____  NO____

Material chute must have a minimum of 7 open positions.

YES____  NO____

The lever system must include a lock device to provide a positive hold in any open position.

YES____  NO____

The opening in the tailgate must be plasma-cut to minimize warping.

YES____  NO____

Material chute door plate must be a minimum thickness of 3/16".

YES____  NO____

(For Tailgate heights of less than 42")

Material chute shall have fully open minimum dimensions of 8" wide by 13" high (no exceptions)

YES____  NO____

Material chute design must allow for the dispensing of small amounts of material without opening the entire tailgate.

YES____  NO____

Material chute must have a minimum of 3 open positions.

YES____  NO____

The lever system must include a lock device to provide a positive hold in any open position.

YES____  NO____

The opening in the tailgate must be plasma-cut to minimize warpage.

YES____  NO____

Material chute door plate must be a minimum thickness of 3/16".

TAILGATE LOCKING MECHANISM & REAR BUMPER DESIGN:

YES____  NO____

Tailgate shall be ??? (air or manual) operated.

YES____  NO____

(If "air operated tailgate" (non asphalt tail): Tailgate air cylinder shall be a minimum 3.5" diameter with a minimum 1" diameter nitrided and chromed shaft (no exceptions).

YES____  NO____

(If "air operated tailgate" (asphalt tail): Tailgate air cylinder shall be a minimum 3.25" diameter with a minimum 1" diameter nitrided and chromed shaft (no exceptions).

YES____  NO____

(If "manual operated tailgate" (no asphalt tail allowed): Tailgate control shall be designed to operate from the left front corner of the dump body.  A minimum 1" round cross rod shall be used for the lever action at the front of the body.  The cross rod shall be connected to the rear latches with a 3/4" connecting link fastened with pipe supports under the dump body floor and must not protrude below the rear body corners.  All bearing blocks shall be constructed with enough clearance to eliminate the necessity for grease.

YES____  NO____

Tailgate latching mechanism shall be of over-center design.

YES____  NO____

Locking mechanism shall be zinc chromated (no exceptions to protect against corrosion) and individually adjustable.

YES____  NO____

Cross shaft shall be a minimum of 1.25" in diameter.

YES____  NO____

Mechanism pivot bearings and locking device thread shall be greasable.

YES____  NO____

Latch fingers shall be a minimum 5/8" thick steel that seat into a latch holder of a minimum thickness of 3/8" steel. (Remove this specification if "asphalt tail")

YES____  NO____

Locking mechanism pins shall be a minimum 1.25" in diameter.

YES____  NO____

Entire mechanism shall be mounted to a minimum 4 gauge, 80,000 psi yield strength high tensile steel bumper.

YES____  NO____

The bumper shall be full width and shall extend and span the width under the rear posts (no exceptions).  The bumper shall be fully welded to the floor and rear post support extensions to form a high strength structurally sound unit.

REAR POST DESIGN:

YES____  NO____

Rear posts shall be one-piece construction and fabricated out of a minimum of 3/16" 44W steel.

YES____  NO____

Rear posts shall be capped by a 1/4", minimum 80,000 psi yield strength steel support extension that is formed and has a formed 90 degree flange that serves as the side end caps for the bumper assembly(no exceptions).

Note:

- Full Depth rear posts are not acceptable due to their inferior design.  They do not have the structural integrity and reinforcement necessary.

YES____  NO____

Rear posts shall have a small access hole in bottom cap for electrical wire passage.

YES____  NO____

Light hole cutouts shall be enclosed and protected inside the post

ASPHALT TAIL (For tailgate heights of 42” or more ONLY) (Remove all if asphalt tail not desired):

YES____  NO____

Asphalt tail shall have a rear post angle of no more than 77 degrees as measured from the floor (13 degree slope).

YES____  NO____

The rear apron/bumper shall have a 52 degree angle sloping downwards from the back of the body floor and sloping back towards the front of the body.

YES____  NO____

The latch fingers shall be a minimum ¾” flame cut steel.

YES____  NO____

The latch fingers shall be greasable.

HOIST:

YES____  NO____

Hoist Design Type: Low mounting front telescopic with fully nitrided cylinder surface (No Exceptions)

YES____  NO____

Hoist Brand: Mailhot (No Exceptions)

YES____  NO____

Hoist Series: ??? (G3, M, CS or CS-DA - The CS series are for applications involving potential exposure to salt)

YES____  NO____

Hoist Diameter: ???" (Depends on series of hoist, length and capacity of the body)

YES____  NO____

Hoist Stroke: ???" (Depends on body length & rear configuration)

YES____  NO____

Hoist Warranty: Minimum ??? years (Minimum 4 years if G series is specified, 2 years if a M, CS or CS-DA is specified)

REAR HINGE ASSEMBLY: (Must choose one of the following)

YES____  NO____

24 ton rear hinge assembly shall be constructed of 4"x4"x5/16" structural angle with 1 7/8" x 5 ¼” tempered steel hinge pins and 2" thick x 7 7/8” tempered steel hinge blocks/pivots.  Fixed hinge blocks shall be 1” thick x 4 ¼”.  Bearing points shall be greasable.

YES____  NO____

35-45 ton rear hinge assembly shall be constructed of 4"x4"x3/8" structural angle with 2 1/4" x 7 1/8” tempered steel hinge pins and 3" thick x 7 5/8” tempered steel hinge blocks/pivots.  Fixed hinge blocks shall be 1 ½” thick x 4”.  Bearing points shall be greasable.

YES____  NO____

Pivot blocks are adjustable with a bolt on cap for easy pin replacement.

HOIST CRADLE DESIGN: (Must choose one of the following)

YES____  NO____

24-35 ton cradle shall be 32 3/8" wide by 13" deep.

45 ton cradle shall be 32 3/8” wide by 14” deep.

YES____  NO____

Cradle frame shall consist of front and back frame rails of a minimum of 4"x2" x 1/4" structural tubing.

YES____  NO____

The cylinder base mounting blocks shall be a minimum of 2" wide and shall have bolted on caps for easy cylinder replacement.

HYDRAULICS & CONTROLS:

YES____  NO____

Dealer to add info...

LIGHTING & ELECTRICAL:

YES____  NO____

Dealer to add info...

BODY PREPARATION & PRIMER:

YES____  NO____

Body shall be sandblasted.

YES____  NO____

Body shall be primed with a 2-part epoxy primer at a minimum thickness of 4mm (No Exceptions)

YES____  NO____

Primer salt spray resistance must be a minimum of 1,500 hours (No Exceptions - primer manufacturer's literature showing the required minimum salt spray resistance must be submitted with the bid).

PAINT:

YES____  NO____

Dealer to add info...

WELDING:

YES____  NO____

All body welds shall be 100% full and continuous.

DUMP BODY WARRANTY:

YES____  NO____

Minimum two years parts and labour.