BEAU-ROC MODEL MPHD - DUMP BODY SPECIFICATIONS

DUMP BODY SPECIFICATIONS:

GENERAL:

Each bidder must indicate on the following items whether their bid is in compliance or not with the stated specifications.  Exceptions to the bid must be detailed on a separate written sheet using company letterhead.  Bidder must supply literature of the model being bid.  Successful bidder must consult with the supervisor on specifications before fabricating the body(ies).

MAKE & MODEL:

YES____  NO____

BEAU-ROC MPHD (Multipurpose Heavy Duty)

COMPLETION:

YES____  NO____

The unit(s) must be completed and delivered within ??? weeks after receipt of chassis.

DUMP BODY DIMENSIONS:

YES____  NO____

Length: ???'  (Options include 15' to 30' long, usually in 6" increments but 1" increments are acceptable as well)

YES____  NO____

Floor Width: 86" at the front of the body & 88" at the rear of the body

The body must be tapered to assist in load breakaway (no exceptions).

YES____  NO____

Side Height: ???" (Options include 60", 66", 72" & 78", 84” & 96”)

YES____  NO____

Tailgate Height: ???" (Must be the same as side height)

DUMP BODY MATERIALS:

Floor:  ???

YES____  NO____

(Please choose one of the following:

Minimum 1/4" Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength)

Minimum 3/8" Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength))

YES____  NO____

Front: 	Minimum 7 gauge Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength)

YES____  NO____

Sides: 	Minimum 7 gauge Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength)

YES____  NO____

Tailgate:  5/32" Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength))

Longitudinals:

YES____  NO____

Minimum 8" x 18.4lb I-beam structural steel with an outside support flange to support the floor with as wide a span as possible.

BODY UNDERSTRUCTURE DESIGN

YES____  NO____

Body shall be cross-memberless.  Cross-member understructure not acceptable, no exceptions.

YES____  NO____

All seams shall be 100% fully welded.

Body/Safety Prop:

YES____  NO____

A single body prop that meets the OSHA requirement shall be provided with the body.

FLOOR DESIGN:

(For body lengths of 20’ & under):

YES____  NO____

Shall be one-piece (no exceptions - multi-piece floors not acceptable).  Floor sheet shall be formed such that the floor bends up at 57 degrees to join the side sheet forming a knee brace.  The floor shall have a minimum flat width of 75’’ at front of the body and 77’’ at rear of the body.  All seams shall be 100% fully welded.

(For body lengths of more than 20’):

The floor shall have a minimum flat width of 75’’ at front of the body and 77’’ at rear of the body.  All seams shall be 100% fully welded.  Floor sheet to side sheet transition shall be constructed with a knee brace at angles upward from the floor to the side at 57 degrees.

FRONT DESIGN:

YES____  NO____

Front Bulkhead shall be constructed of 2 overlapping pieces.  It shall have a formed recess, 12 1/4" deep by 17 1/4" wide, to house the trunnion mount hoist.  The formed recess shall have chamfered 45deg corners on the two front edges and the two rear edges to reduce stress points (no exceptions).

YES____  NO____

Front Bulkhead shall be designed with 7.5"x7.5" 45 degree front corners for reduced wind resistance and to aid in avoiding exhaust interference issues (no exceptions).

YES____  NO____

Front sheet to floor transition shall be a welded piece of steel at approximately 45 degrees for easy cleanout.

YES____  NO____

The steel used for the transition shall be a minimum of 7 gauge Hardox 450 steel (180,000 psi yield strength and 215,000 psi tensile strength) (no exceptions).

SIDES DESIGN:

YES____  NO____

Sides design shall include a formed side sheet plus a separate structural steel top rail & a separate formed rub rail.

YES____  NO____

Side shall be formed with a two-bend pressed-in horizontal brace to increase rigidity (no exceptions).

Notes:

- "Weld-on horizontal bracing is not acceptable due to inferior design, more weight, and decreased flexibility of the side"

- "Flat sided designs are not acceptable due to inferior rigidity and stability of the side plate"

YES____  NO____

Sides shall taper down to a height of 60” near the front of the body to allow for the clearance necessary for a truck mounted crane.

YES____  NO____

Side taper shall begin downwards at 58” from the front sheet and shall finish at 60” high at 34” from the front sheet.  Please refer to the following schematic as an example to follow.

YES____  NO____

Lower rub rail shall be a minimum of 3/16” high tensile steel, formed and welded to the side sheet and the floor.  The rub rail shall dirt shedding.  It shall have an angle of 40 degrees and shall have a 90 degree end face of 3" high (rub rails that finish in a point or rounded edge are not acceptable).

YES____  NO____

Top rail shall be a 4"x4"x1/4" minimum thickness structural tube.

YES____  NO____

Each side shall consist of one front post, minimum 10 gauge steel and one rear post of one-piece construction and fabricated out of a structural tube of 12”x8”x3/8”.

YES____  NO____

All body seams must be 100% fully welded.

TAILGATE DESIGN:

YES____  NO____

Tailgate shall be of manual side opening design (“Barn Door Style”).

YES____  NO____

Tailgate wear surface shall be a single piece, fully formed, with one pressed-in horizontal brace to increase rigidity. Flat panel designs are unacceptable due to inferior rigidity and stability.

YES____  NO____

Tailgate outer frame shall consist of single, dirt shedding, formed upper and lower brace.  Outer side bracing shall be 3"x4"x1/4" structural tubing.

Note:

- Weld-on intermediate horizontal and/or vertical braces are not acceptable due to the decrease in tailgate plate's resistance to deformation.

YES____  NO____

Side opening hardware shall consist of three side opening hinges with 1 1/4” diameter x 16 ½” long hinge pins.

YES____  NO____

Hinge pins shall have two grease zerks per pin.

YES____  NO____

Tailgate latch shall be equipped with an ergonomic handle and must have a spring-loaded return for safety.

YES____  NO____

All body seams must be 100% fully welded.

HOIST:

YES____  NO____

Hoist Design Type: Low mounting front telescopic with fully nitrided cylinder surface (No Exceptions)

YES____  NO____

Hoist Brand: Mailhot (No Exceptions)

YES____  NO____

Hoist Series: ??? (G3, CS or CS-DA - The CS series are for applications involving potential exposure to salt)

YES____  NO____

Hoist Diameter: ???" (Depends on series of hoist, length and capacity of the body)

YES____  NO____

Hoist Stroke: ???" (Depends on body length & rear configuration)

YES____  NO____

Hoist Warranty: Minimum ??? years (Minimum 4 years if G series is specified, 2 years if a CS or CS-DA is specified)

REAR HINGE ASSEMBLY: (Must choose one of the following)

YES____  NO____

35-45 ton rear hinge assembly shall be constructed of 4"x4"x3/8" structural angle with 2 1/4" x 7 1/8” tempered steel hinge pins and 3" thick x 7 5/8” tempered steel hinge blocks/pivots.  Fixed hinge blocks shall be 1 ½” thick x 4”.  Bearing points shall be greasable.

YES____  NO____

50-55 ton rear hinge assembly shall be constructed of 5"x5"x3/8" structural angle with 2 1/4" x 8 1/8” tempered steel hinge pins and 4" thick x 9 ¾” tempered steel hinge blocks/pivots.  Fixed hinge blocks shall be 1 ½” thick x 5”.  Bearing points shall be greasable.

YES____  NO____

Pivot blocks are adjustable with a bolt on cap for easy pin replacement.

HOIST CRADLE DESIGN: (Must choose one of the following)

YES____  NO____

24-35 ton cradle shall be 32 3/8" wide by 13" deep.

45 ton cradle shall be 32 3/8” wide by 14” deep.

55 ton cradle shall be 32 3/8” wide by 15” deep.

YES____  NO____

Cradle frame shall consist of front and back frame rails of a minimum of 4"x2" x 1/4" structural tubing.

YES____  NO____

The cylinder base mounting blocks shall be a minimum of 2" wide and shall have bolted on caps for easy cylinder replacement.

HYDRAULICS & CONTROLS:

YES____  NO____

Dealer to add info...

LIGHTING & ELECTRICAL:

YES____  NO____

Dealer to add info...

BODY PREPARATION & PRIMER:

YES____  NO____

Body shall be sandblasted.

YES____  NO____

Body shall be primed with a 2-part epoxy primer at a minimum thickness of 4mils (No Exceptions)

YES____  NO____

Primer salt spray resistance must be a minimum of 1,500 hours (No Exceptions - primer manufacturer's literature showing the required minimum salt spray resistance must be submitted with the bid).

PAINT:

YES____  NO____

Dealer to add info...

WELDING:

YES____  NO____

All body welds shall be 100% full and continuous.

DUMP BODY WARRANTY:

YES____  NO____

Minimum one year parts and labour.