BEAU-ROC MODEL MPMD - DUMP BODY SPECIFICATIONS

DUMP BODY SPECIFICATIONS:

GENERAL:

Each bidder must indicate on the following items whether their bid is in compliance or not with the stated specifications.  Exceptions to the bid must be detailed on a separate written sheet using company letterhead.  Bidder must supply literature of the model being bid.  Successful bidder must consult with the supervisor on specifications before fabricating the body(ies).

MAKE & MODEL:

YES____  NO____

BEAU-ROC MPMD (Multi-Purpose, Medium Duty)

COMPLETION:

YES____  NO____

The unit(s) must be completed and delivered within ??? weeks after receipt of chassis.

DUMP BODY DIMENSIONS:

YES____  NO____

Length: ???'  (Options include 10’ to 18' long, usually in 6" increments but 1" increments are acceptable as well)

YES____  NO____

Floor Width: Minimum 87" (no exceptions)

YES____  NO____

Exterior Body Width: Shall not exceed 96” (no exceptions)

YES____  NO____

Side Height: ???" (Options include 42", 48", 54” & 60”)

YES____  NO____

Tailgate Height: ???" (Options include 16” (quick release drop-down), 32”, 42”, 48”, 54” & 60” – Standard is Tailgate at same height as side height)

YES____  NO____

Cabshield: ???" (Options include 12" to 48" in 6" increments)

DUMP BODY MATERIALS:

YES____  NO____

Floor:  ???

(Please choose one of the following:

(Standard Floor Material)

Minimum 3/16" Domex 100W high tensile steel (weather resistant steel - no exceptions, 100,000 psi yield strength & 115,000 psi tensile strength)

(Optional)

Minimum 1/4"  AR225 (65,000 psi yield strength & 105,000 psi tensile strength)

Minimum 5/32" Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength)

Minimum 7 gauge Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength)

1/4" Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength))

YES____  NO____

Front: 	Minimum 3/16” Domex 100W high tensile steel (weather resistant steel - no exceptions, 100,000 psi yield strength & 115,000 psi tensile strength)

YES____  NO____

Sides: 	Minimum 3/16” Domex 100W high tensile steel (weather resistant steel - no exceptions, 100,000 psi yield strength & 115,000 psi tensile strength)

YES____  NO____

Tailgate:  Minimum 11 gauge Domex 100W high tensile steel (weather resistant steel - no exceptions, 100,000 psi yield strength & 115,000 psi tensile strength)

BODY UNDERSTRUCTURE DESIGN

YES____  NO____

Body shall be cross-memberless.  Cross-member understructure not acceptable, no exceptions.

YES____  NO____

Longitudinals shall be C-channel structural steel with a formed flared support flange.  Support flange must have a minimum 1 3/4" returnto support the floor with as wide a span as possible.  Longitudinal shall have a minimum bottom surface span of 2 1/4" wide.

YES____  NO____

All seams shall be 100% fully welded.

YES____  NO____

Longitudinal Height: ???" (8", 10" & 12”)

FLOOR DESIGN:

YES____  NO____

Floor sheet shall be formed such that the floor bends up at 57 degrees to join the side sheet forming a knee brace.  The floor shall have a minimum flat section width of 77’.  All seams shall be 100% fully welded.

FRONT DESIGN:

YES____  NO____

Front Bulkhead shall be of one-piece flat panel design.

YES____  NO____

Front shall be capped at the top with a 4”x3”x3/16” structural tube that runs the full width of the front sheet.

CABSHIELD DESIGN (Optional):

YES____  NO____

Cabshield shall be constructed of 11 gauge steel Domex 100W high tensile steel (weather resistant steel - no exceptions, 100,000 psi yield strength & 115,000 psi tensile strength) .  Cabshield must be 100% fully welded (stitch welding not acceptable).

SIDES DESIGN:

YES____  NO____

Fully formed, single piece side with two press-in horizontal V-braces to increase rigidity.

YES____  NO____

Lower rub rail shall be formed and integral to the side sheet (weld-on rub rails are not acceptable), dirt shedding and joined to the floor.  It shall have an angle of 40 degrees and shall have a 90 degree end face of 3" high (rub rails that finish in a point or rounded edge are not acceptable).

YES____  NO____

4"x3"x3/16” top rail is must be a structural steel HSS tubing.  Formed top rails are not acceptable.

Notes:

- "Weld-on horizontal bracing is not acceptable due to inferior design, more weight, and decreased flexibility of the side"

- "Flat sided designs are not acceptable due to inferior rigidity and stability of the side plate"

YES____  NO____

Rear posts shall be full depth, one-piece construction and fabricated out of a minimum 3/16" steel.

(Optional)

YES____  NO____

Passenger side shall be constructed with a 52” side access door at 14” from the front of the body.

YES____  NO____

Side door shall be constructed of a minimum 3/16” Domex 100W high tensile steel (weather resistant steel - no exceptions, 100,000 psi yield strength & 115,000 psi tensile strength).  Side door frame to be constructed with 4”x3”x3/16” structural HSS tubing.

YES____  NO____

Side door shall be constructed with two press-in horizontal V-braces and integral dirt shedding rub rail.

YES____  NO____

Side door shall be hinged to rotate open towards the back of the body.

Locking mechanism must pivot and lock quickly for ease of operation.  A safety chain must be incorporated as secondary safety mechanism to ensure that the lock does not open accidentally.

YES____  NO____

The locking handle and mechanism shall be incorporated in a hinged steel flap.  The steel flap must be designed to prevent leakage at the hinge pivot point and also serve to keep the door firmly in place when closed.

YES____  NO____

All body seams must be 100% fully welded.

TAILGATE DESIGN:

(Please choose one of the following):

1.

Tailgate shall be of manual side opening design (“Barn Door Style”).

YES____  NO____

Tailgate wear surface shall be a single piece, fully formed, with two press-in horizontal V-braces to increase rigidity. Flat panel designs are unacceptable due to inferior rigidity and stability. Outer side bracing shall be 3”x1”x1/8” structural HSS tubing.

YES____  NO____

Side opening hardware shall consist of two (for 42” & 48” high T/Gs) or three (for 54” & 60” T/Gs) side opening hinges with 1” diameter hinge pins.  The pins shall be inserted in two bushings complete with castle nut.

YES____  NO____

Hinges shall be greasable.

YES____  NO____

Tailgate latch shall be equipped with an ergonomic handle and must have a spring-loaded return for safety.

2.

(For Tailgates of all heights except 16”)

YES____  NO____

Tailgate shall be of double manual side opening design (“Double Barn Door Style”). Tailgate wear surfaces shall be a single piece, fully formed, with two (2x) pressed-in horizontal V-braces to increase rigidity. Flat panel designs are unacceptable due to inferior rigidity and stability. Tailgate outer frame shall consist of a single, dirt shedding, formed 3/16” thick upper and lower brace.  Outer side bracing shall be 4”x3”x3/16” structural HSS tubing.

YES____  NO____

Tailgate shall consist of two center bottom-mounted, spring-loaded latches plus one safety spring-loaded latch mounted center and 18” up from the bottom of the door.  Tailgate consists of an inner overlap plate to prevent debris and material from spilling out.

3.

(For 16” drop down tailgate with optional double upper manual side opening doors)

YES____  NO____

Tailgate shall be of quick-release, drop down design and must have the ability to lay flat so that the surface of the tailgate is flush with the surface of the floor.  Mechanism consists of one central pivot handle mounted on a bearing with greasable pins that are activated simultaneously.

YES____  NO____

(Optional – available with 16” only) – Tailgate shall have a mesh (expanded metal) upper double barn door.  Mesh frame shall be designed with a 2”x2”x3/16” frame and shall be locked in place by a spring latch.

YES____  NO____

Sufficient length of 3/8" Grade 70 chains for the full function of the tailgate.

YES____  NO____

All tailgate hinge linkage must have grease zerks at all pivot points.

YES____  NO____

All body seams must be 100% fully welded.

TAILGATE LOCKING MECHANISM & REAR BUMPER DESIGN:

YES____  NO____

Latch fingers shall be a minimum 5/8" thick steel that seat into a latch holder of a minimum thickness of 3/8" steel.

YES____  NO____

Locking mechanism pins shall be a minimum 1.25" in diameter.

YES____  NO____

Entire mechanism shall be mounted to a minimum 3/16”, 80,000 psi yield strength high tensile steel bumper.

YES____  NO____

The bumper shall be full width and shall extend and span the width under the rear posts (no exceptions).  The bumper shall be fully welded to the floor and full depth rear posts to form a high strength structurally sound unit.

REAR POST DESIGN:

YES____  NO____

Rear posts shall be full depth, one-piece construction and fabricated out of a minimum of 3/16" 44W steel.

ROOF DESIGN (Optional):

YES____  NO____

Dump body shall be equipped with a removable roof.

YES____  NO____

Roof shall be ??” long by 12” high.

YES____  NO____

Roof shall be made of ?? (one or two) section(s).

YES____  NO____

Roof shall be constructed of 11 gauge Domex 100W high tensile steel (weather resistant steel - no exceptions, 100,000 psi yield strength & 115,000 psi tensile strength).

YES____  NO____

Roof shall be designed to be fastened with multiple Buyers TGL3410ST Safety Twist Locks.

YES____  NO____

Roof frame shall be a constructed of 3”x1”x1/8” structural HSS tubing.

YES____  NO____

Roof must design must be made with structural bends to increase structural rigidity and that also allows for proper water drainage. (Refer to the following 3D CAD design pictorial)

UNDERBODY HOIST:

Hoist Design Type: Dealer to add details…

YES____  NO____

Hoist Brand: Dealer to add details…

YES____  NO____

Hoist Series: Dealer to add details…

YES____  NO____

Hoist Diameter: Dealer to add details…

YES____  NO____

Hoist Stroke: Dealer to add details…

YES____  NO____

Hoist Warranty: Dealer to add details…

REAR HINGE ASSEMBLY: (Must choose one of the following or add your own hinge info)

YES____  NO____

24 ton rear hinge assembly shall be constructed of 4"x4"x5/16" structural angle with 1 7/8" x 5 ¼” tempered steel hinge pins and 2" thick x 7 7/8” tempered steel hinge blocks/pivots.  Fixed hinge blocks shall be 1” thick x 4 ¼”.  Bearing points shall be greasable.

YES____  NO____

35-45 ton rear hinge assembly shall be constructed of 4"x4"x3/8" structural angle with 2 1/4" x 7 1/8” tempered steel hinge pins and 3" thick x 7 5/8” tempered steel hinge blocks/pivots.  Fixed hinge blocks shall be 1 ½” thick x 4”.  Bearing points shall be greasable.

YES____  NO____

Pivot blocks are adjustable with a bolt on cap for easy pin replacement.

HYDRAULICS & CONTROLS:

YES____  NO____

Dealer to add info...

LIGHTING & ELECTRICAL:

YES____  NO____

Dealer to add info...

BODY PREPARATION & PRIMER:

YES____  NO____

Body shall be sandblasted.

YES____  NO____

Body shall be primed with a 2-part epoxy primer at a minimum thickness of 4mil(No Exceptions)

YES____  NO____

Primer salt spray resistance must be a minimum of 1,500 hours (No Exceptions - primer manufacturer's literature showing the required minimum salt spray resistance must be submitted with the bid).

PAINT:

YES____  NO____

Dealer to add info...

WELDING:

YES____  NO____

All body welds shall be 100% full and continuous.

DUMP BODY WARRANTY:

YES____  NO____

Minimum one year parts and labour.