BEAU-ROC MODEL Ultra - DUMP BODY SPECIFICATIONS

DUMP BODY SPECIFICATIONS:

GENERAL:

Each bidder must indicate on the following items whether their bid is in compliance or not with the stated specifications.  Exceptions to the bid must be detailed on a separate written sheet using company letterhead.  Bidder must supply literature of the model being bid.  Successful bidder must consult with the supervisor on specifications before fabricating the body(ies).

MAKE & MODEL:

YES____  NO____

BEAU-ROC Ultra (Premium One-ton, Contractor Style Body)

COMPLETION:

YES____  NO____

The unit(s) must be completed and delivered within ??? weeks after receipt of chassis.

DUMP BODY DIMENSIONS:

YES____  NO____

Length: ???'  (Options include 8’-6” to 18' long, usually in 6" increments but 1" increments are acceptable as well)

YES____  NO____

Floor Width: Minimum 88" (no exceptions).

YES____  NO____

Side Height: ???" (Options include 12", 15", 18", 20")

YES____  NO____

Tailgate Height: ???" (Options include 20", 24", 28")

YES____  NO____

Cabshield: ???" (Options include 12" to 24" in 6" increments)

DUMP BODY MATERIALS:

YES____  NO____

Floor:  ???

(Please choose one of the following:

Minimum 3/16" 44W (44,000 psi yield strength & 80,000 psi tensile strength)

Minimum 1/4"  AR225 (65,000 psi yield strength & 105,000 psi tensile strength)

Minimum 7 gauge Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength)

1/4" Hardox 450 (no exceptions) (180,000 psi yield strength & 215,000 psi tensile strength))

YES____  NO____

Front: 	Minimum 11 gauge Domex 100W (weather resistant steel - no exceptions) (100,000 psi yield strength & 115,000 psi tensile strength)

YES____  NO____

Sides: 	Minimum 11 gauge Domex 100W (weather resistant steel - no exceptions) (100,000 psi yield strength & 115,000 psi tensile strength)

YES____  NO____

Tailgate:  Minimum 11 gauge Domex 100W (weather resistant steel - no exceptions) (100,000 psi yield strength & 115,000 psi tensile strength)

YES____  NO____

Longitudinals: ???

(Please choose one of the following:

For Bodies with a length of less than 8’-6” to 12'-6"

Shall be 5” C-channel mounted on a 4” C-channel subframe.

For Bodies with a length of 8'-6" to 12'-6"

Shall be 8” C-channel.

For Bodies with a length of greater than 12’-6”

Shall be 10” C-channel with a support flange.

BODY UNDERSTRUCTURE DESIGN

YES____  NO____

Body shall be cross-memberless.  Cross-member understructure not acceptable, no exceptions.

YES____  NO____

All seams shall be 100% fully welded.

YES____  NO____

Longitudinal Height: ???" (Options include 8", 9", 10", 12”)

Body/Safety Prop:

YES____  NO____

A single body prop that meets the OSHA requirement shall be provided with the body.

FLOOR DESIGN:

YES____  NO____

Shall be one-piece (no exceptions - multi-piece floors not acceptable).

FRONT DESIGN:

(Please choose one of the following)

YES____  NO____

(For 8T Hoists, 3.5” diameters)

Front Bulkhead shall be constructed of 2 overlapping pieces.  It shall have a formed recess, 6 1/8” deep by 11 5/8" wide, to house the trunnion mount hoist.  The formed recess shall have chamfered 45deg corners on the two rear edges to reduce stress points (no exceptions).

(For 24T & 35T Hoists, 4.5” & 5.5” diameters)

Front Bulkhead shall be constructed of 2 overlapping pieces.  It shall have a formed recess, 12 1/4" deep by 17 1/4" wide, to house the trunnion mount hoist.  The formed recess shall have chamfered 45deg corners on the two rear edges to reduce stress points (no exceptions).

CABSHIELD DESIGN:

YES____  NO____

Cabshield shall be constructed of Domex 100W (weather resistant steel - no exceptions) (100,000 psi yield strength & 115,000 psi tensile strength).

YES____  NO____

Cabshield shall be constructed of a minimum of 11 gauge steel.

YES____  NO____

Cabshield must be 100% fully welded (stitch welding not acceptable).

SIDES DESIGN:

YES____  NO____

Sides shall be formed from one sheet of steel with an integral formed top rail and formed dirt shedding rub rail.

(For Quick Drop Sides)

YES____  NO____

The body shall be furnished with (single or split) quick drop sides on (both or the passenger or the driver) side(s).

The quick release mechanism shall be a single spring loaded handle and rod system mounted on the drop side panel.

YES____  NO____

YES____  NO____

Each side shall consist of one front post, minimum 10 gauge steel and one rear post of one-piece construction and fabricated out of a minimum 44W (44,000 psi yield strength & 80,000 psi tensile strength) 3/16" steel.

YES____  NO____

Board pockets shall be 2 3/16" wide & fabricated with a minimum 10 gauge steel.

YES____  NO____

All body seams must be 100% fully welded.

TAILGATE DESIGN:

YES____  NO____

Tailgate wear surface shall be a single piece, fully formed, with one dirt shedding lower brace.

YES____  NO____

Tailgate shall be furnished with a quick release mechanism.  The top hinge shall be able to be released with a single handle mounted on the tailgate.

YES____  NO____

Top mount hinge shall have a 9" offset.

YES____  NO____

Tailgate shall be double-acting.

YES____  NO____

Sufficient length of 3/8" Grade 70 chains for the full function of the tailgate.

YES____  NO____

All tailgate hinge linkage must have grease zerks at all pivot points.

YES____  NO____

All body seams must be 100% fully welded.

TAILGATE LOCKING MECHANISM & REAR BUMPER DESIGN:

YES____  NO____

Tailgate shall be ??? (air or manual) operated.

YES____  NO____

(If "air operated tailgate": Tailgate air cylinder shall be a minimum 3.5" diameter with a minimum 1" diameter nitrided and chromed shaft (no exceptions).

YES____  NO____

(If "manual operated tailgate" (no asphalt tail allowed): Tailgate control shall be designed to operate from the left front corner of the dump body.  A minimum 1" round cross rod shall be used for the lever action at the front of the body.  The cross rod shall be connected to the rear latches with a 3/4" connecting link fastened with pipe supports under the dump body floor and must not protrude below the rear body corners.  All bearing blocks shall be constructed with enough clearance to eliminate the necessity for grease.

YES____  NO____

Tailgate latching mechanism shall be of over-center design.

YES____  NO____

Locking mechanism shall be zinc chromated (no exceptions to protect against corrosion) and individually adjustable.

YES____  NO____

Cross shaft shall be a minimum of 1.25" in diameter.

YES____  NO____

Mechanism pivot bearings and locking device thread shall be greasable.

YES____  NO____

Latch fingers shall be a minimum 5/8" thick steel that seat into a latch holder of a minimum thickness of 3/8" steel.

YES____  NO____

Locking mechanism pins shall be a minimum 1.25" in diameter.

YES____  NO____

Entire mechanism shall be mounted to a minimum 3/16”, 80,000 psi yield strength high tensile steel bumper.

YES____  NO____

The bumper shall be full width and shall extend and span the width under the rear posts (no exceptions).  The bumper shall be fully welded to the floor and rear post support extensions to form a high strength structurally sound unit.

REAR POST DESIGN:

YES____  NO____

Rear posts shall be one-piece construction and fabricated out of a minimum of 3/16" 44W (44,000 psi yield strength & 80,000 psi tensile strength) steel.

YES____  NO____

Rear posts shall have a small access hole in bottom cap for electrical wire passage.

YES____  NO____

Light hole cutouts shall be enclosed and protected inside the post

HOIST:

YES____  NO____

Hoist Design Type: Low mounting front telescopic with fully nitrided cylinder surface (No Exceptions)

YES____  NO____

Hoist Brand: Mailhot (No Exceptions)

YES____  NO____

Hoist Series: ??? (G3, M, CS or CS-DA - The CS series are for applications involving potential exposure to salt)

YES____  NO____

Hoist Diameter: ???" (Depends on series of hoist, length and capacity of the body)

YES____  NO____

Hoist Stroke: ???" (Depends on body length & rear configuration)

YES____  NO____

Hoist Warranty: Minimum ??? years (Minimum 4 years if G series is specified, 2 years if a M, CS or CS-DA is specified)

REAR HINGE ASSEMBLY: (Must choose one of the following)

YES____  NO____

8 ton rear hinge assembly shall be constructed of 3"x3"x3/16" structural angle with 1 1/4” tempered steel hinge pins and 2" thick tempered steel hinge blocks/pivots.  Fixed hinge blocks shall be 1” thick x 4 ¼”.  Bearing points shall be greasable.

YES____  NO____

24 ton rear hinge assembly shall be constructed of 4"x4"x5/16" structural angle with 1 7/8" x 5 ¼” tempered steel hinge pins and 2" thick x 7 7/8” tempered steel hinge blocks/pivots.  Fixed hinge blocks shall be 1” thick x 4 ¼”.  Bearing points shall be greasable.

YES____  NO____

35-45 ton rear hinge assembly shall be constructed of 4"x4"x3/8" structural angle with 2 1/4" x 7 1/8” tempered steel hinge pins and 3" thick x 7 5/8” tempered steel hinge blocks/pivots.  Fixed hinge blocks shall be 1 ½” thick x 4”.  Bearing points shall be greasable.

YES____  NO____

Pivot blocks are adjustable with a bolt on cap for easy pin replacement.

HOIST CRADLE DESIGN:

YES____  NO____

Cradle shall be 32 3/8" wide by 9" deep.

YES____  NO____

Cradle frame shall consist of front and back frame rails of a minimum of 4"x2" x 1/4" structural tubing.

YES____  NO____

The cylinder base mounting blocks shall be a minimum of 2" wide and shall have bolted on caps for easy cylinder replacement.

HYDRAULICS & CONTROLS:

YES____  NO____

Dealer to add info...

LIGHTING & ELECTRICAL:

YES____  NO____

Dealer to add info...

BODY PREPARATION & PRIMER:

YES____  NO____

Body shall be sandblasted.

YES____  NO____

Body shall be primed with a 2-part epoxy primer at a minimum thickness of 4mm (No Exceptions)

YES____  NO____

Primer salt spray resistance must be a minimum of 1,500 hours (No Exceptions - primer manufacturer's literature showing the required minimum salt spray resistance must be submitted with the bid).

PAINT:

YES____  NO____

Dealer to add info...

WELDING:

YES____  NO____

All body welds shall be 100% full and continuous.

DUMP BODY WARRANTY:

YES____  NO____

Minimum one year parts and labour.